enable

configure terminal

hostname IZMIR

interface serial 0/0/0

ip address 1.0.0.1 255.0.0.0

no shutdown

exit

interface serial 0/0/1

ip address 2.0.0.1 255.0.0.0

no shutdown

exit

interface FastEthernet 0/0

ip address 20.0.0.1 255.0.0.0

no shutdown

exit

router rip

version 2

no auto-summary

network 1.0.0.0

network 2.0.0.0

network 20.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

enable

configure terminal

hostname BURSA

interface serial 0/0/0

ip address 3.0.0.1 255.0.0.0

no shutdown

exit

interface serial 0/0/1

ip address 1.0.0.2 255.0.0.0

clock rate 125000

no shutdown

exit

interface FastEthernet 0/0

ip address 30.0.0.1 255.0.0.0

no shutdown

exit

router rip

version 2

no auto-summary

network 1.0.0.0

network 3.0.0.0

network 30.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

enable

configure terminal

hostname ADANA

interface serial 0/0/0

ip address 4.0.0.2 255.0.0.0

clock rate 125000

no shutdown

exit

interface serial 0/0/1

ip address 5.0.0.2 255.0.0.0

no shutdown

exit

interface FastEthernet 0/0

ip address 40.0.0.1 255.0.0.0

no shutdown

exit

router rip

version 2

no auto-summary

network 4.0.0.0

network 5.0.0.0

network 40.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

enable

configure terminal

hostname KAYSERI

interface serial 0/0/0

ip address 4.0.0.1 255.0.0.0

no shutdown

exit

interface serial 0/0/1

ip address 6.0.0.2 255.0.0.0

clock rate 125000

no shutdown

exit

interface FastEthernet 0/0

ip address 50.0.0.1 255.0.0.0

no shutdown

exit

router rip

version 2

no auto-summary

network 4.0.0.0

network 6.0.0.0

network 50.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

enable

configure terminal

hostname ISTANBUL

interface serial 0/0/0

ip address 10.0.0.1 255.0.0.0

clock rate 125000

no shutdown

exit

interface serial 0/0/1

ip address 2.0.0.2 255.0.0.0

clock rate 125000

no shutdown

exit

interface serial 0/1/0

ip address 3.0.0.2 255.0.0.0

clock rate 125000

no shutdown

exit

router rip

version 2

no auto-summary

network 10.0.0.0

network 2.0.0.0

network 3.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

enable

configure terminal

hostname ANKARA

interface serial 0/0/0

ip address 10.0.0.2 255.0.0.0

no shutdown

exit

interface serial 0/0/1

ip address 5.0.0.1 255.0.0.0

clock rate 125000

no shutdown

exit

interface serial 0/1/0

ip address 6.0.0.1 255.0.0.0

no shutdown

exit

router rip

version 2

no auto-summary

network 10.0.0.0

network 5.0.0.0

network 6.0.0.0

exit

enable password abc.123

exit

copy running-config  startup-config

-----------------------------------------------------------------------------------------------------

| pc | IP | Mask | Ağ geçidi |
| --- | --- | --- | --- |
| Pc-0 | 20.0.0.2 | 255.0.0.0 | 20.0.0.1 |
| Pc-1 | 30.0.0.2 | 255.0.0.0 | 30.0.0.1 |
| Pc-3 | 40.0.0.2 | 255.0.0.0 | 40.0.0.1 |
| Pc-4 | 50.0.0.2 | 255.0.0.0 | 50.0.0.1 |